EEL 4742 LABORATORY EXPERIMENT # 3

Section:	  0011		 Points:  100

Name: 	Kelsey Cameron

Introduction:

The purpose of this lab was to understand how to convert from ASCII to both other ASCII values and other numbers, and to integrate this into assembly.  This lab also taught us how to program in assembly step by step, like how certain functions must be repeated within a loop.  Also, we learned how to sort ASCII values and use conditional statements within assembly, as well as manipulating arrays.  This assignment was quite extensive and covered more concepts than any other lab thus far, therefore the code below will be more helpful in understanding than any description I could provide.

Complete C Code:

/*

* main.c

*/

#include "msp430fg4618.h"

// uncomment this to submit

// comment this to submit

//#include "msp430f5529.h"

#include "stdio.h"

void Init_UART(void);

void printOpposite(char c);

void OUTA_UART(unsigned char A);

unsigned char getASCIIValue(char c);

unsigned char getHexVal(char c);

void part4();

void part5();

unsigned char INCHAR_UART(void);

void printHexValue(char c);

void printUART(char* firstletter);

//  prints one character at a time via the UART from a string of characters to

//the hyperterminal display. This function should continue printing characters until the

//null character is reached (0x00) indicating an end of a string.

int main(void) {

WDTCTL = WDTPW + WDTHOLD;		// passes the locking password to the watchdog and then holds it.

Init_UART();

for(;;){// 110 sets both P2 bit 1 and P2 bit 2 to an output

// Part 1 - uncomment to test												; kelsey cameron

/*

volatile unsigned char first = INCHAR_UART();

volatile unsigned char second = INCHAR_UART();

first = getASCIIValue(first);

second = getASCIIValue(second);

first = first <<4;

OUTA_UART(first+second);

OUTA_UART(0x0A);

OUTA_UART(0x0D);

*/

// Part 2 - uncomment to test

//volatile unsigned char c = INCHAR_UART();

//printHexValue(c);

// Part 3 - uncomment to test

//printOpposite(INCHAR_UART());

//OUTA_UART(0x0A);

//OUTA_UART(0x0D);

// Part 4 - uncomment to test					kelsey cameron

//part4();

// Part 5 - uncomment to test

//part5();

}

}

void part4(){

int b=0;

int first = 0;	// stores the first asci code

int y = 0;

char a;			// stores the binary char

int second = 0;	// stores the second asci code

for(y=0;y<8;y++){

a = INCHAR_UART();

OUTA_UART(a);

a=a-0x30;		// subtracts 30 to convert it to a 1 or 0

a=a<<(7-y);		// shifts the first over by 7, the second over by 6, etc

b=b|a;			// ors with b to make sure the binary is in the correct order

}

OUTA_UART(0x0A);			//newline

OUTA_UART(0x0D);

first= b & 0xF0;		// look at only the first four bits

second= b & 0x0F;		// look at only the last four bits

first=first>>4;			// shift first by 4 right so the extra zero goes away

// converts the actual letters and numbers to asci letters and numbers

if(first>=0xA && first <=0xF)

{

first+=0x37;

}

else if (first>=0x0 && first<=0x9)

{

first+=0x30;

}

if(second>=0xA && second<=0xF)

{

second+=0x37;

}

else if (second>=0x0 && second<=0x9)

{

second+=0x30;

}

// prints

OUTA_UART(first);

OUTA_UART(second);

OUTA_UART(0x0A);			//newline

OUTA_UART(0x0D);

}

void printOpposite(char c){

if(c >= 0x41 && c <= 0x5A){

c+=0x20;

}

else if(c >= 0x61 && c <= 0x7A){

c-=0x20;

}

OUTA_UART(c);

}

void printHexValue(char c){

char second = c & 0x0F;		//0x15 = 1111

char first = c;

first = c & 0xF0;

first = first >> 4;

OUTA_UART(getHexVal(first));

OUTA_UART(getHexVal(second));

}

// 3F = 3*16 + F

unsigned char getASCIIValue(char c){

if(c >= 0x30 && c <= 0x39){

c-=0x30;

}

else if(c >= 0x41 && c <= 0x46){

c-=0x37;

}

return c;

}

unsigned char getHexVal(char c){

if(c >= 0x00 && c <= 0x09){

c+=0x30;

}

else if(c >= 0x0A && c <= 0x0F){

c+=0x37;

}

return c;

}

void part5(){

char array[33];

int num = 0;

int x = 0;

for(x = 0; x < 32; x++){

char c = INCHAR_UART();

OUTA_UART(c);

if((c == '\0') || (c == 0x0D) || (c==0x0A) || (int)(c==10)){

break;

}

num++;

array[x] = c;

}

OUTA_UART(0x0D);

int i, j, temp;

for(i = 0; i < num; i++){

for(j = 0; j<num; j++){

int num1 = array[i];

int num2 = array[j];

if(num1 < num2){

temp = array[i];

array[i] = array[j];

array[j] = temp;

}

}

}

for(i = 0; i < num; i++){

if(array[i]==0x0D || array[i]==0x0A || array[i]=='\0'){

break;

}

OUTA_UART(array[i]);

}

}

void printUART(char * firstletter){

while(*firstletter!='\0'){

OUTA_UART(*firstletter);

firstletter++;

}

OUTA_UART(0x0D);

}

// 1.0 SW1 1.2 SW2

// remember that 0 means button is pressed.

// This code below is for the lab chip.  Uncomment it before submitting, and delete your own chip uart code

void OUTA_UART(unsigned char A){

//---------------------------------------------------------------

//***************************************************************

//---------------------------------------------------------------

// IFG2 register (1) = 1 transmit buffer is empty,

// UCA0TXBUF 8 bit transmit buffer

// wait for the transmit buffer to be empty before sending the

// data out

do{

}while ((IFG2&0x02)==0);

// send the data to the transmit buffer

UCA0TXBUF =A;

}

// while bit 2 of the IFG2 register is set to zero,

// this is a transmitter flag that sees if the transmitter is free to transfer or not.

// wait for the transmission to be complete.  This flag = 1 when bits are on the output lines.

// send the data to the transmit buffer

// so this takes the data from A, and sends it

// the buffer that stores data.  This UCA0TXBUF is a variable

// that stores the address of a buffer register on the chip.

// this data transfer is hard wired, and data from A is placed on the

// output lines that are placed into the register.

// basically UART works by shifting the bits right, and the rightmost bit is sent to the reciever

unsigned char INCHAR_UART(void){

do{

}while((IFG2&0x01) == 0);		// IFG2 = Interrupt FlaG Second Register

// First bit of IFG2 recieves, and the second bit transfers.

return (UCA0RXBUF);

}

void Init_UART(void){

//---------------------------------------------------------------

// Initialization code to set up the uart on the experimenter

// board to 8 data,

// 1 stop, no parity, and 9600 baud, polling operation

//---------------------------------------------------------------

P2SEL=0x30; // transmit and receive to port 2 bits 4 and 5

// sets Bits p2.4 transmit and p2.5 receive, similar to how we set bits as outputs

// or inputs

// 30 = 110000

UCA0CTL0=0; // 8 bits of data, no parity 1 stop, uart, async

// (7)=1 (parity), (6)=1 Even, (5)= 0 lsb first,

// (4)= 0 8 data / 1 7 data,

// (3) 0 1 stop 1 / 2 stop, (2-1) -- UART mode,

// (0) 0 = async

// this basically initializes the flag.

UCA0CTL1= 0x41;

// select ALK 32768 and put in

// software reset the UART

// (7-6) 00 UCLK, 01 ACLK (32768 hz), 10 SMCLK,

// 11 SMCLK

// (0) = 1 reset

UCA0BR1=0; // upper byte of divider clock word

UCA0BR0=3; // clock divide from a clock to bit clock 32768/9600

// = 3.413

// UCA0BR1:UCA0BR0 two 8 bit reg to from 16 bit

// clock divider

// for the baud rate

UCA0MCTL=0x06;

// low frequency mode module 3 modulation pater

// used for the bit clock

UCA0STAT=0; // do not loop the transmitter back to the

// receiver for echoing

// (7) = 1 echo back trans to rec

// (6) = 1 framing, (5) = 1 overrun, (4) =1 Parity,

// (3) = 1 break

// (0) = 2 transmitting or receiving data

UCA0CTL1=0x40;

// take UART out of reset

IE2=0; // turn transmit interrupts off

//---------------------------------------------------------------

//***************************************************************

//---------------------------------------------------------------

// IFG2 register (0) = 1 receiver buffer is full,

// UCA0RXIFG

// IFG2 register (1) = 1 transmit buffer is empty,

// UCA0RXIFG

// UCA0RXBUF 8 bit receiver buffer

// UCA0TXBUF 8 bit transmit buffer

}

C Code Screenshots:

Complete Assembly Code:

; final assembly code for lab #3 kelsey cameron

.cdecls C,LIST,"msp430fg4618.h"

.sect ".const"

.bss label, 4

.word 0x1234

.text

.global _START

.bss myarray, 32

;----------------------------------------------------------------

START 		mov.w #300h,SP

StopWDT 	mov.w #WDTPW+WDTHOLD,&WDTCTL

; uncomment the lab stuff above this line

; delete below this line for lab

;-------------------------------------------------------------------------------

; MSP430 Assembler Code Template for use with TI Code Composer Studio

;

;

;-------------------------------------------------------------------------------

;            .cdecls C,LIST,"msp430f5529.h"       ; Include device header file

;-------------------------------------------------------------------------------

;           .def    RESET                   ; Export program entry-point to

; make it known to linker.

;-------------------------------------------------------------------------------

;          .text                           ; Assemble into program memory.

;          .retain                         ; Override ELF conditional linking

; and retain current section.

;           .retainrefs                     ; And retain any sections that have

; references to current section.

;-------------------------------------------------------------------------------

;RESET       mov.w   #__STACK_END,SP         ; Initialize stackpointer

;StopWDT     mov.w   #WDTPW|WDTHOLD,&WDTCTL  ; Stop watchdog timer

;;;;;;;;;;;;;;;;;;;

call #Init_UART

Mainloop

;call #part1																													; kelsey cameron

;call #part2

;call #part3

;call #part4

call #part5

EXIT jmp Mainloop

part1

call #INCHAR_UART

call #OUTA_UART

mov.b R4, R7	; R7 will represent first

call #INCHAR_UART

call #OUTA_UART

mov.b R4, R8	; R7 will represent second

;	call #OUTA_UART

convertFirst		cmp.b #0x30, R7

jge next11

next11			cmp.b #0x39, R7

jl addop1

cmp.b #0x39, R7

jeq addop1

jmp next21

addop1		sub.b #0x30, R7

jmp cont

next21		cmp.b #0x41, R7

jge next31

next31		cmp.b #0x46, R7

jl subtract1

next51		cmp.b #0x46, R7

jeq subtract1

subtract1	sub.b #0x37, R7

jmp cont

cont

convertSec		cmp.b #0x30, R8

jge next112

next112			cmp.b #0x39, R8

jl addop12

cmp.b #0x39, R8

jeq addop12

jmp next212

addop12		sub.b #0x30, R8

jmp cont2

next212		cmp.b #0x41, R8

jge next312

next312		cmp.b #0x46, R8

jl subtract12

next512		cmp.b #0x46, R8

jeq subtract12

subtract12	sub.b #0x37, R8

jmp cont2

cont2

rla R7

rla R7

rla R7

rla R7

mov.b #0x0D, R4

call #OUTA_UART

mov.b #0x0A, R4

call #OUTA_UART

add.b R7, R8

mov.b R8, R4

call #OUTA_UART

Exit1 ret

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

; input S output 53

part2

;char second = c & 0x0F;		//0x15 = 1111

;	char first = c;

;first = c & 0xF0;

;first = first >> 4;

;OUTA_UART(getHexVal(first));

;OUTA_UART(getHexVal(second));

call #INCHAR_UART

call #OUTA_UART

mov.b R4, R7

and.b #0xF0, R7

mov.b R4, R8

and.b #0x0F, R8

rra R7

rra R7

rra R7

rra R7

checkLetter	cmp.b #0x0A, R7

jge letter1

jmp checkNum

letter1		cmp.b #0x0F, R7

jge EXIT3

jmp letter2

letter2 	add.b #0x37, R7

jmp shiftb

checkNum	cmp.b #0x0, R7

jge number1

jmp EXIT3

number1		cmp.b #0x9, R7

jge Mainloop

jmp number2

number2 	add.b #0x30,R7

jmp shiftb

shiftb		mov.b R7, R4

;call #OUTA_UART

NEXT		cmp.b #0x0A, R8

jge letter3

jmp checkNum2

letter3		cmp.b #0x0F, R8

jge EXIT3

jmp letter4

letter4 	add.b #0x37, R8

jmp PRINT

checkNum2	cmp.b #0x0, R8

jge number3

jmp EXIT3

number3		cmp.b #0x9, R8

jge EXIT3

jmp number4

number4 	add.b #0x30,R8

jmp PRINT

PRINT		mov.b R7, R4

call #OUTA_UART

mov.b R8, R4

call #OUTA_UART

EXIT3 ret

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

part3

call #INCHAR_UART

call #OUTA_UART

CHECKUPPER		cmp.b #0x41, R4

jge next1

next1			cmp.b #0x5A, R4

jl addop

cmp.b #0x5A, R4

jeq addop

jmp next2

addop		add.b #0x20, R4

call #OUTA_UART

jmp Exit

next2		cmp.b #0x61, R4

jge next3

next3		cmp.b #0x7A, R4

jl subtract

next5		cmp.b #0x7A, R4

jeq subtract

subtract	sub.b #0x20, R4

call #OUTA_UART

jmp Exit

Exit ret

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

; part4

part4

mov.b #0x0, R10			; the counter for the loop

mov.b #0x07, R11				; shift amount for each character

mov.b #0x0, R12			; R12 stores the binary number

mov.b #0x0, R4

mov.b #0x0, R13

mov.b #0x0, R15

loop

cmp.b #0x08, R15

jge exitloop

call #INCHAR_UART

mov.b R4, R7				; R7 now stores each character

call #OUTA_UART			; echo back what you recieve

sub #0x30, R7		; subtract 30 from R7 to change ascii value into 1 or 0

; R7 contains number that you need to shift

mov.b #0x0, R10

shift

cmp.b R11, R10			; compare counter to shift amount

jge stopshift			; stop shifting if R10 = R11

rla R7, #0x01				; shift the 1 or zero over based on how many times you've looped

add #0x01, R10				; increment counter by 1

jmp shift

stopshift

add R7, R12				; add the 1 or zero to whatever was previously stored in R12

add #0x01, R15

sub #0x01, R11

jmp loop

exitloop

mov.b R12, R13	; R12 will store upperhalf of number, R13 stores lower half.

and.b #0xF0, R12		; first bits

and.b #0x0F, R13		; second bits

rra R12

rra R12

rra R12

rra R12				; shifts the bits in R12 to the right 4 times to get rid of zeros

convert			cmp.b #0x0A, R12

jge checkNext

jmp checkNext2

checkNext		cmp.b #0x0F, R12

jl addConvert

cmp.b #0x0F, R12

jeq addConvert

jmp checkNext2

addConvert		add.b #0x37, R12

jmp dontadd				; finish and compare second num

checkNext2		cmp.b #0x00, R12

jge checkNext3

jmp dontadd

checkNext3		cmp.b #0x09, R12

jl convertAdd

cmp.b #0x09, R12

jeq convertAdd

jmp dontadd

convertAdd		add.b #0x30, R12

dontadd

cmp.b #0x0A, R13

jge checkNext23

jmp checkNext22

checkNext23		cmp.b #0x0F, R13

jl addConvert2

cmp.b #0x0F, R13

jeq addConvert2

jmp checkNext22

addConvert2		add.b #0x37, R13

jmp finish2

checkNext22		cmp.b #0x00, R13

jge checkNext33

jmp finish2

checkNext33		cmp.b #0x09, R13

jl convertAdd2

cmp.b #0x09, R13

jeq convertAdd

jmp finish2

convertAdd2 		add.b #0x30, R13

; Now you've converted everything and you can print

finish2 mov.w R12, R4

call #OUTA_UART

mov.w R13, R4

call #OUTA_UART

mov.w #0x0A, R4

call #OUTA_UART

mov.w #0x0D, R4

call #OUTA_UART

Exit2 ret

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

part5

;get or set first element myarray(0)

mov.b #0x0D, R9

mov.b #0x00, R8

mov.b #32, R7		; value you will count to

mov.b #0x0, R4

mov.b #0x0, R14		; a temporary variable to store the asci char

mov.b #0x0, R6		; counter for array buffer

loop77

cmp.b R7, R6

jge stoploop77

call #INCHAR_UART

mov.b R4, R8				; R8 now stores each character

cmp.b R8, R9					; if enter is pressed, stop taking in chars

jeq stoploop77

mov.b R8, myarray(R6)		; move character

mov.b myarray(R6), R4

call #OUTA_UART

add.b #0x01, R6				; increment counter

jmp loop77

stoploop77

mov.b R9, R4				; prints enter

call #OUTA_UART

;;;;;;;;;;;;;; now do the sorting

; R6 contains the # you count to.  Use R7 as counter

mov.b #0x0, R7		; R7 will be the counter for loopfirst

; R8 will be the counter for loopsecond

mov.b #0x0, R8

loopfirst

cmp.b R6, R7

jeq stoploopfirst

mov.b #0x0, R8

loopsecond

cmp.b R6, R8

jeq stoploopsecond

;;;;;;;;; the switch

mov.b myarray(R7), R15

mov.b myarray(R8), R15

;;; delete these two above

cmp.b myarray(R7), myarray(R8)

jge switch

jmp dontswitch

switch

mov.b myarray(R7), R14					; R14 = R7

mov.b myarray(R8), myarray(R7)			; R7 = R8

mov.b R14, myarray(R8)					; R8 = R14

;

dontswitch

;;;;;;;;;;;;;

add.b #0x01, R8

jmp loopsecond

stoploopsecond

add.b #0x01, R7

jmp loopfirst

stoploopfirst

; R7 is loop counter to print array

; R6 stores # of chars

mov.b #0x0, R7

printArray

cmp.b R8, R7

jge stopprinting

mov.b myarray(R7), R4

call #OUTA_UART

add.b #0x1, R7

jmp printArray

stopprinting

Exit7 ret

; LAB UART CODE UNCOMMENT TO TEST IN Lab

OUTA_UART

;----------------------------------------------------------------

; prints to the screen the ASCII value stored in register 4 and

; uses register 5 as a temp value

;----------------------------------------------------------------

; IFG2 register (1) = 1 transmit buffer is empty,

; UCA0TXBUF 8 bit transmit buffer

; wait for the transmit buffer to be empty before sending the

; data out

push R5

lpa 	mov.b &IFG2,R5

and.b #0x02,R5

cmp.b #0x00,R5

jz lpa

;	; send the data to the transmit buffer UCA0TXBUF = A;

mov.b R4,&UCA0TXBUF

pop R5

ret

INCHAR_UART

push R5

lpb 	mov.b &IFG2,R5

and.b #0x01,R5

cmp.b #0x00,R5

jz lpb

mov.b &UCA0RXBUF,R4

pop R5

;go get the char from the receive buffer

ret

Init_UART

;----------------------------------------------------------------

;Initialization code to set up the uart on the experimenter board to 8 data,

; stop, no parity, and 9600 baud, polling operation

;----------------------------------------------------------------

mov.b #0x30,&P2SEL

mov.b #0x00,&UCA0CTL0

mov.b #0x41,&UCA0CTL1

mov.b #0x00,&UCA0BR1

mov.b #0x03,&UCA0BR0

mov.b #0x06,&UCA0MCTL

mov.b #0x00,&UCA0STAT

mov.b #0x40,&UCA0CTL1

mov.b #0x00,&IE2

ret

;----------------------------------------------------------------

; Interrupt Vectors

;----------------------------------------------------------------

.sect ".reset" ; MSP430 RESET Vector

.short START ;

.end

Assembly Screenshots:

Conclusion:

In conclusion, this experiment taught me a lot about ASCII values.  Prior to this, I didn’t realize that you could add and subtract characters like numbers in Hex, and I never really realized that  a character is stored in hex within the chip.  I learned a lot about syntax for assembly in this experiment (like how I constantly forgot to write .b after almost everything, and how I often forgot the pound signs), but this extra experience has improved my ability to program in assembly code and enlightened my understanding of what is actually going on within the hardware.